Bacnet Javascript (Node.js) Information

- It is basically a bacnet stack written in ‘C’ and wrapped for Node.js

- Can be downloaded from: https://www.npmjs.com/package/bacnet

- Checked for working properly on Linux System

- Installation Procedure:

  - Open Terminal

  - Type and run: npm install bacnet

- Directory structure of installed stack is as follows:

When installed on Odroid, the installed stack is shown as per above image.

## Contents

- bacnet-stack/ - contains the full contents of the c library checked out from svn://svn.code.sf.net/p/bacnet/code/trunk/bacnet-stack

  - .svn/ - the svn db is checked in to make it easier to update to newer versions and in case we have something to push back to the c library - I don't know if this is a bad idea

- src/ - contains the wrapper code

  - h_*.c - request and response message handlers - modified from code in bacnet-stack/demo/handlers

  - functions.cc - contains the addon code that runs when functions are called from js

  - init.cc - runs on module initialisation

  - module.cc - defines the module

  - *conversion.cc - provides conversions between js and the BACnet structs

  - emitter.cc - emits js events into an EventEmitter in js-land asynchronously with events from the c code

- examples/ - contains example script which uses the module

Implemented Functions:

- whois

- write property

- read property

Implemented handle to receive these messages:

- i am

- read ack

- write ack

For detailed information, visit: https://www.npmjs.com/package/bacnet

How to run examples

Server must be downloaded and installed from link as: https://sourceforge.net/projects/bacnetserver/

Run the server as shown in below image.

After the server is started, following screen will appear.

Device ID of server

IP of server

Open terminal window and enter examples directory as below:

Open and edit the example file to be run. For ex. To edit example named readProoperty.js do as follows:

Edit the file as highlighted in figure below.

Device ID must be of server

IP must be of server

Now run the example as per below image:

Node-RED Implementation

After the stack is installed, Run node-red on the device.

[{"id":"5446abb8.1fb4f4","type":"inject","z":"946af227.86ee5","name":"","topic":"","payload":"","payloadType":"date","repeat":"","crontab":"","once":true,"x":261,"y":66,"wires":[["f4b39d6.b88946"]]},{"id":"f4b39d6.b88946","type":"unsafe-function","z":"946af227.86ee5","name":"Launch_Object_device","func":"'use strict'\nconst async = require('async')\nconst bacnet = require('bacnet')\nconst r = bacnet.init({\n  datalink: {\n    iface: 'wlan0',\n    ip_port: 0xBAC0\n  },\n  device: true\n})\n\n\nr.whois('141.219.219.6',260001,260003)\nr.on('iam', function (iam) {\n  console.log('iam: ', iam)\n\n})\n","outputs":1,"noerr":0,"x":497,"y":58,"wires":[[]]},{"id":"8532c69f.3820f8","type":"unsafe-function","z":"946af227.86ee5","name":"Read_property","func":"const bacnet = require('bacnet')\nconst r = bacnet.init({\n  datalink: {\n    iface: 'wlan0',\n    ip_port: 0xBAC0\n  },\n  device: false\n})\n\nfunction withAddressOrId (addressOrId, callback) {\n  if (addressOrId.match(/^\\d+$/)) { // integer - should be a device Id so we have to do a whois\n    console.log('doing whois for ' + addressOrId)\n    r.whois(Number(addressOrId))\n    r.on('iam', function (iam) {\n      console.log('iam: ', iam)\n      callback(Number(addressOrId))\n    })\n  } else { // something else - should be an address so we dont need to do a whois\n    callback(addressOrId)\n  }\n}\n\nfunction objectIdToString (objectId) {\n  return bacnet.objectTypeToString(objectId.type) + '/' + objectId.instance\n}\n\nconsole.log('reading property', bacnet.propertyKeyToString('object-list'))\n\nwithAddressOrId('141.219.219.6', function (addressOrId) {\n  r.readProperty('141.219.219.6', 'device', '260002', 'object-list', false, function (err, property) {\n    if (err) throw console.log('Error', err)\n    console.log('Received property /', objectIdToString(property.object), '/', bacnet.propertyKeyToString(property.property))\n    console.log(property.value)\n  })\n})\n\nsetTimeout(function () {}, 1000)\n","outputs":1,"noerr":0,"x":479,"y":209,"wires":[[]]},{"id":"32c52be6.05af44","type":"inject","z":"946af227.86ee5","name":"","topic":"","payload":"","payloadType":"date","repeat":"","crontab":"","once":false,"x":259,"y":224,"wires":[["8532c69f.3820f8"]]}]

Copy the above code and paste into Node-RED import from clipboard area as in figure below:

Following flow will appear:

Click here

Click on inject button as shown in above figure.

Following text will appear in console window.

JavaScript written in above function node is shown below:

'use strict'

const async = require('async')

const bacnet = require('bacnet')

const r = bacnet.init({

datalink: {

iface: 'wlan0',

ip_port: 0xBAC0

},

device: true

})

r.whois('141.219.219.6',260001,260003)

r.on('iam', function (iam) {

console.log('iam: ', iam)

})

JavaScript written in above function node is shown below:

const bacnet = require('bacnet')

const r = bacnet.init({

datalink: {

iface: 'wlan0',

ip_port: 0xBAC0

},

device: false

})

function withAddressOrId (addressOrId, callback) {

if (addressOrId.match(/^\d+$/)) { // integer - should be a device Id so we have to do a whois

console.log('doing whois for ' + addressOrId)

r.whois(Number(addressOrId))

r.on('iam', function (iam) {

console.log('iam: ', iam)

callback(Number(addressOrId))

})

} else { // something else - should be an address so we dont need to do a whois

callback(addressOrId)

}

}

function objectIdToString (objectId) {

return bacnet.objectTypeToString(objectId.type) + '/' + objectId.instance

}

console.log('reading property', bacnet.propertyKeyToString('object-list'))

withAddressOrId('141.219.219.6', function (addressOrId) {

r.readProperty('141.219.219.6', 'device', '260002', 'object-list', false, function (err, property) {

if (err) throw console.log('Error', err)

console.log('Received property /', objectIdToString(property.object), '/', bacnet.propertyKeyToString(property.property))

console.log(property.value)

})

})

setTimeout(function () {}, 1000)

Validation:

Download and install YABE. https://sourceforge.net/projects/yetanotherbacnetexplorer/

Run YABE:

Following screen will appear:

Compare details in below screen with the data in node-RED console window.

Properties

OBJECTS

Write property service implementation

Writeproperty.js

Firstly, get access to your single board computer (odriod/Raspberry pi). Enter the bacnet library by:

$:cd node_modules/bacnet/examples

A sample JavaScript code of Write property.js has been provided. Some modification must be made to make it work. (nano writeProperty.js)

- var value = ‘xxx’

this prepares the property value that we want to write.

- Target host ip address: xxx.xxx.xxx.xxx (for here it is 192.168.1.106).

- Configure the arguments of r.writeProperty function

- Object: ‘device’ (currently, it can only be ‘device’)

- Object number: 260002

- Property (name): ‘object-name’

- Property value: variable value in json format.

After these modification, run this writeProperty.js script by:

$: node writeProperty.js

Create a block in node-red:

Execute node-red in the single board computer by

$: node-red

Open browser, enter xxxx.xxx.xxx.xxx:1880 (here 192.168.1.106:1880) to open the node-red interface,

Use unsafe-function (must be pre-installed) to create a node that has the writeproperty function.

Entering code as follows:

const bacnet = require('bacnet')

const r = bacnet.init({

datalink: {

iface: 'wlan0',

ip_port: 0xBAC0

},

device: false

})

function withAddressOrId (addressOrId, callback) {

if (addressOrId.match(/^\d+$/)) { // integer - should be a device Id so we have to do a whois

console.log('doing whois for ' + addressOrId)

r.whois(Number(addressOrId))

r.on('iam', function (iam) {

console.log('iam: ', iam)

callback(Number(addressOrId))

})

} else { // something else - should be an address so we dont need to do a whois

callback(addressOrId)

}

}

//var value = JSON.parse(1)

var value ='meem502'

console.log('writing property', bacnet.propertyKeyToString('object-name'), 'value', value)

withAddressOrId('192.168.1.104', function (addressOrId) {

r.writeProperty('192.168.1.104','device',260002,'object-name', false, value, function (err) {

if (err) console.log('error', err)

else console.log('success')

})

})

setTimeout(function () {}, 1000)

The object-name of the ‘device’ object will be modified from “1301” to “meem502”